DMA Controller

Features :

- 8 AXI RD/WR channels for Memory operations

- APB interface for CSR access

- Interrupts for completed transfer and Error cases

- SRAM based descriptor

- Descriptor ring manager for managing head/tail and state of the descriptor

Possible DV bugs :

- Incorrect sampling of shared common registers ( for example register to indicate valid descriptors) ,   Scoreboard will wrongly process disabled/invalid  descriptors.

- Descriptor ring manager not configured correctly, for example any overlapping source/destination address or size of transfers etc will result into wrong operations.

- Scoreboard not handling un-aligned accesses correctly

- Incorrect prediction of interrupt by the scoreboard

- Incorrect handling of multiple interrupts happening concurrently

- Incorrect handling of multiple DMA channels active at same time

- Incorrect handling of any AXI Error Response from any of the DMA channels

- Improper Strobe handling while capturing the data from AXI bus

- Handling of boundary addresses for any of the DMA channels , resulting into accessed address going beyond the configured range.